# SDIE V6 — Model Flow

Structural Drawing Intelligence Engine: teach from reference projects (INIZIO, TRUST_OFFICE, MANOHAR), then infer and quantify on any raw DXF using GENERIC merged knowledge. Outputs include slab and beam concrete takeoff.

# 1. End-to-End Flow

Two phases: (A) offline teach from tagged reference DXFs, (B) runtime inference on any drawing.

┌─────────────────────────────────────────────────────────────────┐
    │  A · TEACH (offline)                                            │
    │  Tagged DXFs → Atlas + Layer Profiles + Knowledge Base          │
    └────────────────────────────┬────────────────────────────────────┘
                                 │ merged artifacts
                                 ▼
    ┌─────────────────────────────────────────────────────────────────┐
    │  B · INFER (runtime, project-id GENERIC)                        │
    │                                                                 │
    │  Input DXF                                                      │
    │      → Prepare (layers, floor zone, entities)                   │
    │      → Classify (rules, graph, DeepSeek)                        │
    │      → Quantify (beams + slabs)                                 │
    │      → Export (JSON, overlay, Excel, review queue)            │
    └─────────────────────────────────────────────────────────────────┘

# 2. Teach Pipeline (Phase A)

1. Read projects_manifest.json

2. Load tagged DXF per reference project

3. build_atlas.py → component_atlas.json

4. build_layer_profiles.py → layer_profiles.json

5. RAG / KB builder → structural_kb.json

# 3. Inference Pipeline (Phase B)

| Step | Action |
| --- | --- |
| ① | Load DXF — units, scale, extents |
| ② | Auto-discover layers — frame, annotation, void, column |
| ③ | Resolve floor zone — THK label clusters |
| ④ | Load teach artifacts — Atlas + KB (GENERIC = all projects) |
| ⑤ | Extract entities — structural, cutout, hatch void, columns |
| ⑥ | Classify components — V5: rules → graph → DeepSeek on ambiguous |
| ⑦ | Beam quantities — centerline length × section → concrete m³ |
| ⑧ | Build structural graph — nodes and topology edges |
| ⑨ | Slab detection — exclusions → beam grid → framed bays |
| ⑩ | Slab quantities — area × THK → concrete m³ |
| ⑪ | Export — JSON, overlay, Excel (Slabs + Beams), review queue |

# 4. V5 Classification Logic

Hard global layers → locked type (Opening, Column, Beam, etc.)

Soft layers (S_FRAMES, STR-BEAM) → geometry heuristics first

GENERIC mode → best teach rule per layer across all projects

V5 confidence < 75% → DeepSeek with RAG + graph context

Supporting types excluded from slab area: walls, cores, shafts, openings

# 5. Quantity Engines

| Element | Inputs | Formula |
| --- | --- | --- |
| Beam | Classified beam lines, beam-size tags | length × width × depth → m³ |
| Slab | Framed bays after exclusions, THK labels | area × thickness → m³ |
| Combined | Slab + beam totals | Reported in Summary and Excel |

Slab fallback: beam-grid → region polygonize → beam-frame bbox.

# 6. Inference Mode

| project-id | Knowledge | Use case |
| --- | --- | --- |
| GENERIC (default) | All teach projects merged | New / unknown drawings |
| INIZIO | Inizio + GLOBAL | Teach evaluation |
| TRUST_OFFICE | Trust + GLOBAL | Teach evaluation |

# 7. Key CLI Flags

python scripts/run_pipeline.py <dxf> -o <output_dir>

--project-id GENERIC (default)

--no-deepseek — skip DeepSeek reasoning

--no-beam-quantities — skip beam takeoff

DEEPSEEK_API_KEY in Phase2_Concrete_Estimation/.env

# 8. Output Files

{drawing}_results.json

{drawing}_building_model.json

{drawing}_overlay.html / .svg

{drawing}_quantities.xlsx — Summary, Slabs, Beams, Classification

{drawing}_review_queue.json

{drawing}_summary.txt

SDIE V6 · P2_SlabVersion6 · Teach-then-infer structural estimation